TRACKBEE

PCB Version 3 — Changelog

ESP32-C6 IoT Development Board

| Status | IN PROGRESS |
| --- | --- |
| Base | V2 (SCH_Schematic1_20251201) |
| Date | January 2026 |
| Author | Francois |

Overview : track - overview

Doc track_V3 (old) : track_V3-120126

# 1. Background and objectives

This document defines the modifications to be made between version 2 (V2) and version 3 (V3) of the TrackBee board. It is based on feedback from V2 tests, schematic analysis, BOM analysis, and flying probe analysis.

## Problems identified in V2

- Audible noise / crackling on the +5V rail (TPS54531 area / battery management)

- Single I2C bus shared between all devices → address conflicts and SCL locked at ~1.96V in battery mode

- Inability to operate in deep sleep without +5V (complete dependence on the TPS54531)

- Power path of the BQ25792 not compliant with the TI EVM (VAC1/VAC2 short-circuited, ACDRV not used)

- Back-powering between domains (cross-I2C pull-ups between AON and SYS rails)

- MAX17048 incompatible with 2S battery pack

## V3 main objectives

- Standalone deep sleep on battery— Deep sleep mode must operate WITHOUT the +5V rail, powered directly from the battery via a dedicated 3.3V regulator (3V3_AON)

- Complete remanufacturing of BQ25792— Correction of I2C issues, alignment of the power path with the TI EVM reference design

- Creation of 3 separate I2C buses— I2C_AON, I2C_SYS_SW, I2C_TPS to eliminate conflicts and back-powering

## Components and connectors added in V3

Summary table of everything that is NEW compared to V2 (excluding modifications to existing components).

### Active components added

| Component | Reference | Role V3 |
| --- | --- | --- |
| 3V3_AON Buck | TPS6293D | 3.3V/2A regulator from SYS (3.8–30V). Powers the AON domain in deep sleep, independent of +5V. |
| 2S Fuel gauge | MAX17049 | Replaces MAX17048 (incompatible with 2S). Always-on battery gauge on I2C_AON. |
| wake accelerometer | LIS2DW12 | Low-power sensor dedicated to vibration wake-up (INT1 → LP_GPIO). On I2C_AON, 3V3_AON. |
| VBUS_DET Comparator | To be defined | Ultra-low-power USB detection (~10nA). Low active open-drain output to wired-OR IO7. |
| USB-PD ORing NFC | Back-to-back pair | PPHV → VBUS_BQ via ACDRV1 (ideal diode ORing). Replaces short circuit VAC1 V2. |
| Solar Oring NFCs | Back-to-back pair | SOLAR_IN → VBUS_BQ via ACDRV2 (ideal diode ORing). |
| Solar TVS | To be defined | Overvoltage protection on SOLAR_IN to GND, before the input FETs. |
| MOSFET strike plate | N-ch logic-level (SOT-23) | 5V/2A power stage for solenoid door strike. Gate driven by MCP23017 GPB0. J_STRIKE_SOL output is MOSFET-switched (low-side); MCP output never carries solenoid current. See §6.6. |
| Flyback diode strike | SS34 (Schottky 3A/40V) | 2A solenoid inductive protection. Schottky for quick clamping. SS14 (1A) borderline in thermal performance → SS34 preferred (same footprint SMA/DO-214AC). |

### Connectors added

| Connector | Kind | Role V3 |
| --- | --- | --- |
| Servo motor | JST-XH 3-pin | +5V / GND / PWM header. Powered by switched +5V rail (5V_EN). |
| Power supply strike plate (J_STRIKE_PWR) | JST B2B-XH-A(LF)(SN) | Internal +5V output (TPS54531 switched rail) + GND. 5V/2A pulsed solenoid only. |
| Solenoid strike plate (J_STRIKE_SOL) | JST B2B-XH-A(LF)(SN) | Output to solenoid (blue + white). Switched by MOSFET. |
| J_MCP_GPB (expander) | B3B-XH-A | I/O extension on the GPB lines of the MCP23017 (GPB1–GPB7 available, GPB0 = STRIKE_CTRL). |
| USB-C charge-only | XH 2 pins | Dedicated connector for charging (+/-) only, no data. |
| External RESET | B2B-XH-A(LF) | Replaces the C19 push button with a remote connector. |

### Added assets and networks

| Element | Detail |
| --- | --- |
| Wired-OR IO7 Network | Pull-up 10kΩ (3V3_AON) + 3× series resistors 100–330Ω (VBUS_DET, BTN, RAK_IRQ) |
| Gate latch resistances | R gate 100Ω–1kΩ (MCP GPB0 → gate) + R pull-down 10kΩ–100kΩ (OFF by default, including unpowered MCP) |
| I2C series resistors | 33–100Ω at AON ↔ SYS domain crossings (anti-back-powering) |
| Additional bulk capacities | VIN TPS25750: 4.7µF → 34.7µF (+30µF). P5V rail: 10µF → 310µF (+300µF). |
| 5V_EN pull-down | Pull-down resistor on IO0 (5V_EN) to guarantee +5V OFF at boot and in deep sleep |
| BQ CE# pull-down | 100kΩ pull-down resistor on CE# (pin 13 BQ25792) for safe operation without USB |
| Cap REGN BQ25792 | 4.7µF X5R/X7R 16V on REGN (pin 7). Internal LDO. Required. (ref. EVM C35) |
| Caps BTST1/BTST2 BQ | 2× 47nF C0G/X7R 25V. BTST1→SW1 (pin 25→24) + BTST2→SW2 (pin 28→27). (ref. EVM C6/C8) |
| PROG BQ Network | Resistive divider on PROG (pin 4): 2S / 1.5 MHz / 2A configuration. (ref. EVM R20=255k, R21=127k) |
| R ILIM_HIZ BQ | 100kΩ between ILIM_HIZ (pin 5) and REGN. IINDPM = 1.5A by default. (ref. EVM R23) |
| TS BQ network (optional) | Footprints for direct NTC and network simulation (R24=30.1k, R25=10k, R26=4.87k). Assemble according to the package. |
| Bridge D+/D− BQ | 0Ω or solder bridge between D+ (pin 14) and D− (pin 15). Simulates DCP (1.5A). (ref. EVM JP10) |

# 1bis. V3 Change Checklist — by Priority

Ordered list from most critical to least critical. To be checked off as implementation progresses on the diagram/layout. Reference to detailed sections in parentheses.

| ☐ | # | Modification | Comment / Reply |
| --- | --- | --- | --- |
|  |  | BQ25792 — POWER PATH & BATTERY |  |
| ☐ | 1 | Separate VAC1 (USB-PD) and VAC2 (solar). (§3.3) |  |
| ☐ | 2 | ORing NFET back-to-back via ACDRV1/ACDRV2. (§3.4) |  |
| ☐ | 3 | VBUS BQ pins → VBUS_BQ after ORing FETs. (§3.5) |  |
| ☐ | 4 | CE# (pin 13): 100kΩ pull-down resistor only. NO CHREN. (§3.1) |  |
| ☐ | 5 | QON# (pin 12): pull-down 100kΩ GND (not direct bridge). (§3.2) |  |
| ☐ | 6 | ADCIN1→1V5, ADCIN2→GND. Caps bulk. (§3.8) |  |
| ☐ | 7 | Battery: protection + solar decoupling. (§3.6, §3.7) |  |
| ☐ | 8 | BQ I2C on I2C_AON bus (IO10/IO11). (§3.10) |  |
| ☐ | 9 | PROG (pin 4): 2S / 1.5 MHz / 2A network. (§3.11) |  |
| ☐ | 10 | ILIM_HIZ (pin 5): 100kΩ→REGN, 1.5A default. (§3.12) |  |
| ☐ | 11 | TS (pin 6): NTC footprints + network simulation. (§3.13) |  |
| ☐ | 12 | SDRV (pin 9): NC (no FET ship). (§3.14) |  |
| ☐ | 13 | D+/D− (pins 14/15): DCP short circuit, 0Ω. (§3.15) |  |
| ☐ | 14 | REGN 4.7µF + BTST1/BTST2 47nF as close as possible. (§3.16) |  |
|  |  | POWER & DEEP SLEEP |  |
| ☐ | 15 | TPS6293D (SYS→3.3V AON, Iq ~12µA). (§2.1.1) |  |
| ☐ | 16 | TPS54531 (+5V): IN IO0, pull-down, layout. (§2.1.3, §2.6) |  |
| ☐ | 17 | Domains SYS_SW, +3.8V, LDO_3V3: EN OFF default. (§2.2, §2.5) |  |
| ☐ | 18 | CP2102N on VBUS (VREGIN pin 7). (§2.8) |  |
|  |  | I2C — 3 BUS & ADDRESSES |  |
| ☐ | 19 | 3 buses: I2C_AON, I2C_SYS_SW, I2C_TPS. (§4) |  |
| ☐ | 20 | MCP23017 @ 0x27 (A0=A1=A2=VDD). (§4.2, §4.5) |  |
| ☐ | 21 | LIS2DW12 SA0=VDD, IIS2ICLX SDO=GND. (§4.5, §5.1, §5.2) |  |
| ☐ | 22 | R series 33–100Ω AON↔SYS crossovers. (§4.4) |  |
|  |  | WAKE & GPIO |  |
| ☐ | 23 | Wired-OR IO7: pull-up + 3 active low sources. (§2.3, §8.6) |  |
| ☐ | 24 | VBUS_DET Comparator ultra-low-power. (§2.3) |  |
| ☐ | 25 | STRIKE_CTRL → MCP23017 GPB0, IO2 spare. (§6.6, §8.4) |  |
|  |  | PERIPHERALS & SENSORS |  |
| ☐ | 26 | MAX17048 → MAX17049 (fuel gauge 2S). (§5.3) |  |
| ☐ | 27 | RAK3172 on 3V3_AON, stop2, UART2. (§6.1.3) |  |
| ☐ | 28 | Strike: MOSFET + SS34 + pull-down + JST-SM. (§6.6) |  |
| ☐ | 29 | Servo: JST-XH 3 pins (+5V/GND/PWM IO1). (§6.5) |  |
|  |  | CONNECTORS & LAYOUT |  |
| ☐ | 30 | J_MCP_GPB: B3B-XH-A (GPB1–GPB7). (§6.4) |  |
| ☐ | 31 | RESET (C19) → B2B-XH-A(LF) can be moved. (§6.4) |  |
| ☐ | 32 | USB-C charge-only: XH connector. (§6.4) |  |
| ☐ | 33 | Test points + pads for current measurement. (§2.7) |  |
| ☐ | 34 | Footprints DNP: snubber, preload, bypass. (§2.6) |  |

# 2. Power Architecture and Deep Sleep

## 2.1 Principle: Deep sleep independent of +5V

Deep sleep is the default operating mode. Only the 3V3_AON rail remains powered; all other rails must be physically switched off (EN pin or load switch).

### 2.1.1 3V3_AON Regulator

Source:BQ25792 SYS (VBAT/2S, always present as long as the battery or USB is connected)

Selected regulator:TPS6293D — synchronous buck, VIN 3.8–30V, output 3.3V, 2A max (ref. TI datasheet SNVSCK3)

Justification:Switching (and non-linear) regulator to limit heat dissipation with a 2S input that can reach ~8.4V.

Deep sleep IQ requirement:The no-load current (Iq) of the AON regulator is the dominant factor in the deep sleep budget. The TPS6293D typically draws Iq of ~12µA (TI SNVSCK3 datasheet, VIN=12V, no load). The total AON load in deep sleep (ESP32 ~7µA + LIS2DW12 ~1µA + MAX17049 ~3µA + pull-ups ~5µA) is estimated at ~28–35µA. VIN min = 3.8V: SYS is always > 4V in battery mode (2S Li-ion) → the TPS6293D starts in all cases.

V3 Target:Iconso deep sleep total < 50µA on 3V3_AON rail. Validate by measurement on prototype (AON current measurement pad provided in §2.7).

⚠️ BQ25792 ADC in deep sleep:The BQ25792 consumes ~17–24µA (battery-only, ADC off) but ~540µA if the internal ADC remains active (ref. datasheet SLUSDG1, Table "Typical Application Characteristics"). The firmware MUST disable the BQ's ADC before entering deep sleep: register REG2E[7] ADC_EN = 0. Without this, the 50µA budget cannot be maintained.

Confirmation SYS BQ25792 — SYS is available in all 3 cases:

- Battery present / USB absent: SYS powered by battery via the BQ power path

- USB present (with or without battery): SYS powered by the adapter (USB-PD) and/or battery (NVDC)

- Battery missing: deterministic behavior — the board either operates from USB or remains off

### 2.1.2 Definition of food supply areas

| Rail | Source | Deep sleep state | Load |
| --- | --- | --- | --- |
| 3V3_AON | SYS → TPS6293D | ON (always active) | ESP32 (sleep), LIS2DW12, MAX17049, BQ25792 (I2C), RAK3172 wake, I2C_AON pull-ups |
| 3V3_SYS_SW | +5V → LDO/buck | OFF (ESP32 EN) | microSD, GNSS, IIS2ICLX, MCP23017, I2C_SYS_SW pull-ups |
| TPS_LDO3V3 | TPS25750 internal LDO | ON if VBUS is present | CAT24C256 (EEPROM), I2C_TPS pull-ups. Autonomous dead-battery boot. |
| +5V | SYS → TPS54531 | OFF (EN low) | +5V consumers, 3V3_SYS_SW source, 3.8V GSM |
| +3.8V GSM | +5V → TPS63070 | OFF (IN MCU) | A7670E (modem) |
| LDO_3V3 (legacy) | TLV75533 external | Deleted V3 | Replaced by TPS_LDO3V3 (internal LDO of the TPS25750) for EEPROM + I2C_TPS. The TLV75533 is no longer required. |

### 2.1.3 TPS54531 (+5V) — deactivable in deep sleep

- Pin EN pulled low by default (< 1.25V) → +5V OFF at startup

- ESP32 control via GPIO (logic high 1.5–6V to activate)

- No hysteresis required — ESP32 is the only power supply controller

## 2.2 Explicit sleep killer gating

| Peripheral | Domain | Mechanism | Default state |
| --- | --- | --- | --- |
| microSD (U21) | 3V3_SYS_SW | High-side load switch (VDD_SD) | OFF (SD_EN down) |
| CP2102 (U2) | VBUS direct (VREGIN) | USB powered | OFF except when USB is plugged in |
| TXS0102 (U13) | 3V3_SYS_SW | OE pull-down | OFF (OE down) |
| MAX17049 (U8) | 3V3_AON | Permanent | ON (continuous SoC) |
| MCP23017 (U15) | 3V3_SYS_SW | Rail SYS_SW | OFF deep sleep |

### 2.2.1 Firmware requirements before rail shutdown (GPIO anti back-powering)

Ruler :Before disabling 3V3_SYS_SW (deep sleep entry), the firmware MUST configure all GPIOs connected to the SYS_SW domain as Hi-Z (input, no internal pull). Otherwise, an ESP32 GPIO driven HIGH (3V3_AON) can back-power a SYS_SW device via its internal ESD diodes.

| GPIO | Signal | Action before deep sleep |
| --- | --- | --- |
| IO3 | I2C_SYS_SW_SCL | Input Hi-Z (disable internal pull-up). External pull-up on 3V3_SYS_SW drops with the rail. |
| IO15 | I2C_SYS_SW_SDA | Input Hi-Z (disable internal pull-up). Idem. |
| SPI SD (IO18–21) | SD_CLK, SD_MOSI, SD_MISO, SD_CS | Input Hi-Z. VDD_SD must be cut BEFORE to avoid leakage via MOSI/CLK. |
| IO0 | 5V_EN | Output LOW (disable +5V), then Hi-Z. External pull-down keeps OFF. |

Recommended cut-off order:(1) GPIO SPI SD → Hi-Z, (2) SD_EN = LOW (cut VDD_SD), (3) GPIO I2C_SYS_SW → Hi-Z + disable internal pulls, (4) BQ25792 ADC_EN = 0 (REG2E[7]), (5) 5V_EN = LOW (cut +5V and 3V3_SYS_SW), (6) enter deep sleep.

Objective:Wake the ESP32 from deep sleep when connected via USB, with a pure hardware guarantee (no BQ firmware dependency).

V3 Solution:Discrete VBUS_DET (ultra-low-power comparator ~10nA, low active output) integrated into the wired-OR on IO7 (LP_GPIO7) with BTN_AON and RAK3172_IRQ.

- Wake-on-demand without any prior I2C configuration

- Wake level-based (stable condition as long as VBUS is present)

- State defined when VBUS is absent (no floating state), ESD protection included

- BQ25792 INT# removed as a dedicated wake source — BQ registers are read via I2C_AON after wake-up to retrieve event details

Advantage:Frees up IO3 (formerly BQ_INT) for the I2C_SYS_SW bus. Reduces the number of LP_GPIO wakes to 2 (IO4 + IO7), leaving some headroom.

## 2.4 Deep sleep wake sources

| Source | Signal → ESP32 GPIO | Pull-up | Kind |
| --- | --- | --- | --- |
| LIS2DW12 (vibration) | INT1 → IO4 (LP4) | 3V3_AON | Level |
| Wired-OR (3 sources) | WAKE_OR_N → IO7 (LP7) | 3V3_AON | Level |
| → VBUS_DET_N | USB comparator (active low) | — | (wired-OR) |
| → BTN_N | Push button (GND contact) | — | (wired-OR) |
| → RAK_IRQ_N | RAK3172 open-drain (LoRa) | — | (wired-OR) |
| RESET | Button → reset ESP32 | — | HW reset |
| RTC timer | ESP32 internal | — | Timer |

### 2.4.1 Typical scenario: connecting a USB charger

Initial state:Board in deep sleep (only 3V3_AON active, battery present). The user connects a USB-C charger.

| Stage | Time | Action |
| --- | --- | --- |
| 1 | t=0 | VBUS appears on the USB-C connector. The BQ25792 detects the adapter and activates the NVDC power-path: the battery is charged AND the SYS rail powers the board simultaneously. |
| 2 | t ≈ immediate | VBUS_DET comparator pulls output LOW (active low). Via wired-OR, IO7 goes LOW. |
| 3 | t ≈ qq ms | The ESP32 wakes up from deep sleep (EXT1 wake on IO7 = ANY_LOW). The 3V3_AON was already active. |
| 4 | t ≈ boot | The firmware identifies the wake source: reading BQ25792 via I2C_AON (immediately accessible, no need to enable SYS_SW). The registers indicate "USB adapter detected". |
| 5 | t ≈ boot+ | The firmware decides whether or not to activate the +5V (IO0 = 5V_EN HIGH) according to the application logic. |

Key answer:YES, the USB charger charges the battery AND powers the board simultaneously. This is the NVDC (Narrow Voltage DC) mode of the BQ25792: the internal power path ensures that the SYS rail remains available during charging. Charging starts automatically (CE# = LOW by default via pull-down, see §3.1).

Special case — battery missing:The BQ25792 powers SYS directly from the USB adapter. The board functions normally without a battery. If the USB cable is removed, the board shuts down (no battery backup).

## 2.5 Default states of EN/control pins

- Nonee pin EN/control must remain floating

- Each EN/control: default pull resistance + optional 0Ω strap to MCU GPIO

- Applies to: TPS54531 EN, 3.8V EN rail, EN SYS_SW, and all switched rails

## 2.6 TPS54531: layout rework

Objective:Reduce the switching loop (hot loop) to attenuate audible noise, ringing, EMI.

- Place L3 and D4 as close as possible to TPS54531 SW/PGND pins

- Minimize the SW copper island

- Input capacitors (10µF + 100nF + 10nF) directly to VIN/PGND, shortest loop

- PGND return clean with dense stitching near the exposed pad

- Router VSENSE/FB as quiet signal: taken after L3, far from SW

## 2.7 Measurement and debug points

Current measurement pads: AON rail, SYS_SW rail, +5V rail (cuttable jumpers/pads).

Test points: TPS54531 SW, TPS54531 VIN, +5V, SYS, PPHV, VBAT.

- Optional RC footprint snubber near the switching node (DNP)

- +5V preload resistor footprint for audible noise reduction (DNP)

- 0Ω bypass option in parallel with bead L4

## 2.8 USB programming mode (dead/blank ESP)

Issue :In V3, +5V and 3V3_SYS_SW are OFF by default (5V_EN is low). If the ESP is dead, bricked, or blank, it cannot activate 5V_EN. It must be possible to flash the ESP even in this state.

Solution:The CP2102N is powered directly from VBUS via its VREGIN pin (pin 7), and not from 3V3_SYS_SW.

- The CP2102N has an internal LDO (VREGIN → VDD 3.3V). Connecting it via USB is sufficient to power it.

- The PROG_TX, PROG_RX, RTS, DTR lines to the ESP are fed by the internal VDD of the CP2102.

- The ESP32 in deep sleep is powered by 3V3_AON (always ON if battery is present). In flash mode, the USB connects to VBUS → SYS BQ → 3V3_AON → powered ESP.

- The auto-download circuit (Q1/Q2 RTS/DTR) handles the reset + BOOT0 automatically.

Result :Simply connecting a USB cable is enough to flash the ESP, regardless of the firmware or +5V rail status. No manual intervention is required (except for the BOOT button if auto-download fails).

# 3. Resumption of BQ25792 — EVM TI Alignment

Reference :TI BQ25792EVM User Guide (SLUUCB5E), drawings pages 19–20 —https://www.ti.com/lit/ug/sluucb5e/sluucb5e.pdf

## 3.1 CE# (Load Enable) — pin 13

Problem V2:CE# floating → risk of disabling the load.

V3 Solution:CE# pulled low by default via pull-down 100kΩ to GND. Load always allowed (CE# active low = LOW = ON).

⚠️ Correction (EVM review) — CP2102 CHREN link removed:The previous version planned to connect the CHREN pin of the CP2102N to the CE# pin of the BQ25792. This connection is incorrect: CHREN is active HIGH (HIGH = available USB current, ref. SiLabs AN §4.3.4). Furthermore, BC1.2 detection on the CP2102N is disabled by default and requires prior configuration via Simplicity Studio (Xpress Configurator). Connecting CHREN directly to CE# could have unexpectedly disabled the load.

Truth table (corrected):

| Condition | CE# (BQ pin 13) | Load |
| --- | --- | --- |
| USB plugged in or unplugged (default) | LOW (pull-down 100kΩ) | Activated ✅ |
| ESP firmware inhibits charging | HIGH (ESP GPIO push-pull) | Disabled |
| ESP in deep sleep | LOW (GPIO Hi-Z + pull-down) | Activated ✅ |

Option:If firmware control of the charging process is desired, connect an ESP32 GPIO pin (SYS_SW domain) to CE# via a 1kΩ series resistor. A 100kΩ pull-down resistor ensures charging is enabled during deep sleep, boot, and reset. The firmware can set CE# HIGH to disable charging if necessary (e.g., high temperature, full battery, etc.).

## 3.2 QON# — pin 12

Problem V2:QON# indefinite.

V3 Solution:Pull down 100kΩ to GND (QON# = permanent LOW). Ship mode will never be activated inadvertently.

⚠️ Justification vs direct bridge:The BQ25792 datasheet (SLUSDG1 §9.3.4) describes the exit ship mode via a LOW pulse on QON# for >800ms. A direct GND bridge works (permanent LOW = immediate exit), but a 100kΩ pull-down resistor is safer: it allows adding a button or jumper in the future without breaking the trace, and avoids any ambiguity about the behavior if the ship mode is accidentally activated by the firmware. A footprint of 0402 is required for the R pull-down resistor + optional pad jumper.

## 3.3 Separation VAC1 / VAC2

Problem V2:VAC1 and VAC2 are short-circuited.

- VAC1 (pin 9) = PPHV (USB-PD high voltage)

- VAC2 (pin 8) = SOLAR_IN

- VAC1 and VAC2 must NOT be shorted

## 3.4 ORing / blocking via ACDRV1/ACDRV2

Problem V2:ACDRV1/2 not used, no ideal diode ORing.

USB-PD:PPHV → back-to-back NFETs → VBUS_BQ ; gates by ACDRV1 (pin 11)

Solar:SOLAR_IN → back-to-back NFETs → VBUS_BQ ; gates by ACDRV2 (pin 10)

## 3.5 BQ VBUS pin reconnection

VBUS (pins 2 & 3) → VBUS_BQ (after input FETs), and NOT directly to PPHV.

## 3.6 Sun protection + decoupling

- TVS to GND on SOLAR_IN (optional PTC in series), before the FETs

- Bulk decoupling + HF close to the VBUS/PGND pins of the BQ

- Short PPHV routing, away from sensitive traces, clearance adapted to PD voltage

## 3.7 Battery side

BAT (pins 22/23) and BATP (pin 18) unchanged in V3. External BMS in the pack.

## 3.8 ADCIN Configuration + Capabilities

- ADCIN1 → 1V5, ADCIN2 → GND (dividers to TPS_LDO3V3, ref. SLVSFR7A Table 9-4).

⚠️ Double effect of ADCIN:The ADCIN1/ADCIN2 pins simultaneously select: (1) the I2C address (slave) of the TPS25750 — here index #1 = 0x20, and (2) the Dead Battery Configuration — here "SafeMode" = the TPS applies Rd to CC and can sink 5V/0.5A without a loaded EEPROM patch (minimal USB-PD boot). The other dead-battery options (AlwaysEnableSink, NegotiateHighVoltage) require different ADCIN combinations. Some modes are incompatible with EEPROM patch loading.

- Reference: TPS25750 datasheet SLVSFR7A, Table 9-4 “ADCIN1/ADCIN2 decode” and §9.3.6 “Dead Battery”.

- VIN TPS25750: 4.7µF → 34.7µF

- P5V line TPS25750: 10µF → 310µF

## 3.9 Default Straps

Define the default assembled solder bridges for PPHV / SOLAR_IN / VAC1 / VAC2. The default build must NOT be able to short-circuit VAC1 and VAC2.

## 3.10 I2C BQ25792 — on the AON bus

In V2, the BQ25792 shared the single system I2C bus → source of the problem SCL at 1.96V in battery mode.

V3 Solution:The BQ25792 I2C bus is connected to the I2C_AON bus (3V3_AON pull-ups). The BQ is powered by SYS (always ON) and must be accessible upon waking from deep sleep, before the SYS_SW rails are activated. It is programmable by the MCU via I2Cs (ref. TI SLVUC05A §1.2.1).

## 3.11 PROG (pin 4) — default configuration at reset

Role :The PROG pin sets the charger's default parameters at power-on reset (number of cells, charging current, switching frequency) before any I2C communication.

V3 Choice (2S battery, 1.5 MHz):Resistive network identical to the installed EVM JP24. Voltage divider PROG → AGND selecting 2S / 2A / 1.5 MHz. Ref. datasheet SLUSDG1 Table 9-1.

| Setting | Value | Notes |
| --- | --- | --- |
| Cell count | 2S | Vreg = 8.4V by default (2×4.2V) |
| Ichg default | 2A | Modifiable via I2C after boot |
| Fsw | 1.5 MHz | Inductance L2 sized accordingly |

- Check the R divisor values ​​against Table 9-1 of the datasheet (SLUSDG1). The EVM uses R20=255k and R21=127k for 2S/1.5MHz.

## 3.12 ILIM_HIZ (pin 5) — hardware input current limiting

Role :Sets the maximum input current (IINDPM) by default at power-on, independently of I2C. Also serves as the HIZ (high impedance) pin to cut off the converter.

Option V3 (1.5A by default):100kΩ resistor between ILIM_HIZ and REGN (identical to the installed EVM JP14). Select IINDPM = 1.5A at boot. The firmware can increase this up to 3.3A via I2C if the source (USB-PD) allows it.

- Do NOT leave ILIM_HIZ floating → undefined behavior.

- If we want 500mA by default (standard USB): Direct resistance ILIM_HIZ → GND.

## 3.13 TS (pin 6) — battery thermistor

Role :Battery temperature measurement input via NTC/PTC. The BQ25792 can block, reduce or cut off the charge depending on the measured temperature (TCOLD, TCOOL, TWARM, THOT).

Case 1 — NTC in the battery pack:Connect the 10k NTC thermistor from the battery pack directly to the TS pin. The bias network is integrated into the BQ (REGN/TS/AGND divider). Verify the temperature thresholds using I2C registers.

Case 2 — No NTC (external BMS manages the temperature):Polarize TS to simulate "normal temperature" continuously. Equivalent EVM network: R24=30.1kΩ (REGN→TS) + R25=10kΩ (TS→AGND) + R26=4.87kΩ in parallel. This places TS in the TNORMAL zone of the BQ. Alternatively, disable TS protection via I2C (TS_IGNORE bit, REG10).

- Decision V3: To be confirmed depending on the battery pack used. Footprints should be planned for both cases (direct NTC OR network simulation).

## 3.14 SDRV (pin 9) / Ship FET — transport mode

Role :SDRV drives the gate of a battery-side serial NFET (ship FET / Q5 on EVM). In ship mode, the BQ completely disconnects the system battery (Iq < 1µA). Ship mode is activated via I2C and deactivated by pressing QON#.

Choice V3 — no FET ship:QON# is pulled down via a 100kΩ pull-down resistor (§3.2), so ship mode is not used. The SDRV pin must be left unconnected (NC) and BAT connected directly to the pack (via an external BMS).

- If ship mode is desired in the future: provide footprint NFET (type CSD17575Q3, VSON-8) and QON# button (modify §3.2).

- SDRV = NC. Do not pull to GND (would short-circuit the internal driver).

## 3.15 D+ / D− (pins 14/15) — USB source detection

Role :The D+/D− pins of the BQ25792 allow detection of the USB source type (SDP 500mA, CDP 1.5A, DCP 1.5A, non-standard) via the BC1.2 protocol.

V3 Context:The TPS25750 handles USB-PD negotiation and sets the available power. BC1.2 detection of the BQ is not required for the USB-PD input (VAC1). However, if a basic USB-A charger is connected (without PD), BC1.2 can be useful to allow >500mA.

V3 Choice:Short-circuit D+ and D− together (as with the EVM JP10 installed). This simulates a DCP (Dedicated Charging Port) and allows 1.5A by default without negotiation. The firmware increases IINDPM if USB-PD negotiates more.

- Resistance 0Ω between D+ and D− (or solder bridge, DNP if we want to activate the real detection later).

- Alternative: D+ and D− connected to the USB-C connector via the TPS25750 for full detection. More complex, requires additional routing.

## 3.16 REGN (pin 7) and BTST (pins 25/28) — mandatory capacitors

Role :REGN is the BQ25792 internal LDO (3.3V) powering gate drivers and logic. BTST1/BTST2 are bootstrap for buck-boost high-side FETs.

| Pine | Cape | EVM Ref. | Notes |
| --- | --- | --- | --- |
| REGN (pin 7) | 4.7µF | C35 | X5R/X7R 16V ceramic min. As close as possible to the pin. |
| BTST1 (pin 25) → SW1 | 47nF | C6 | C0G/X7R ceramic 25V min. Between BTST1 and SW1. |
| BTST2 (pin 28) → SW2 | 47nF | C8 | C0G/X7R ceramic 25V min. Between BTST2 and SW2. |

Criticality:These capacitors are mandatory and absent from schematic V2. Without REGN capacitors, the internal LDO is unstable. Without BTST capacitors, the high-side FETs do not switch correctly → no load.

- Placement: as close as possible to the pins, same layer as the BQ25792 (no vias). Ref. PCB Layout Guidelines §3 of the EVM SLUUCB5E guide.

# 4. I2C Architecture — 3 separate buses

Problem V2:A single I2C bus shared by all devices. SCL at ~1.96V in battery mode. Address conflicts and back-powering.

## 4.1 I2C_AON (Always-On)

Pull-ups:3V3_AON

| Component | Address | Role | Supply |
| --- | --- | --- | --- |
| BQ25792 (U3) | 0x6B | Loadur / PMIC | SYS (always ON) |
| MAX17049 (U8) | 0x36 | 2S Fuel gauge | 3V3_AON |
| LIS2DW12 | 0x19 (SA0=VDD) | Vibration wake | 3V3_AON |

## 4.2 I2C_SYS_SW (Switched)

Pull-ups:3V3_SYS_SW (shut down in deep sleep)

| Component | Address | Role | Supply |
| --- | --- | --- | --- |
| MCP23017 (U15) | 0x27 (A0-A2=VDD) | IO expander | 3V3_SYS_SW |
| IIS2ICLX (U22) | 0x6A | Inclinometer | 3V3_SYS_SW |
| TPS25750 (U11) slave | 0x20 (index #1) | USB-PD ctrl | TPS_LDO3V3 |

TPS25750:Connect pins 8 (I2Cs_SDA) and 9 (I2Cs_SCL) to I2C_SYS_SW bus.

⚠️ TPS25750 power supply architecture (dead-battery review):The TPS25750 has an internal LDO (pin LDO_3V3) that activates as soon as VBUS is present, BEFORE any other system rail. This LDO powers the dead-battery boot: the TPS reads its configuration from the EEPROM (CAT24C256) on I2C_TPS to start the USB-PD negotiation. If the EEPROM is on an external rail (3V3_SYS_SW), it is not powered when the USB connection is made → configuration not loaded → PD degraded or disabled.

Final V3 solution:Create a dedicated TPS_LDO3V3 rail (internal LDO output of the TPS25750, pin LDO_3V3). This rail feeds: (1) CAT24C256 (EEPROM), (2) I2C_TPS pull-ups (I2Cm). The I2C_SYS_SW bus (ESP ↔ TPS I2Cs side) maintains its pull-ups on 3V3_SYS_SW — the TPS I2Cs tolerates different levels than I2Cm. Explicitly rename the nets: TPS_LDO3V3 (TPS25750 internal LDO) vs 3V3_SYS_SW (ESP rail). NEVER call either "LDO_3V3".

✅ Address TPS25750 — collision avoided:The I2C address of the TPS25750 is determined by the ADCIN1/ADCIN2 pins (Table 9-4 datasheet SLVSFR7A). The four possible addresses are 0x20, 0x21, 0x22, and 0x23. The MCP23017 is moved to 0x27 (A0=A1=A2=VDD), outside this range → no possible conflict regardless of the ADCIN configuration.

## 4.3 I2C_TPS (dedicated, master-slave)

Topology:Item-to-point TPS25750 (master) ↔ CAT24C256 (slave EEPROM)

Pull-ups:TPS_LDO3V3 (internal LDO of the TPS25750). This rail is active as soon as VBUS is present → EEPROM accessible at dead-battery boot. No back-powering: I2C_TPS is isolated from the I2C_SYS_SW bus.

- TPS25750 I2Cm_SDA (pin 16) ↔ CAT24C256 SDA

- TPS25750 I2Cm_SCL (pin 17) ↔ CAT24C256 SCL

- CAT24C256: A0=A1=A2=GND (address 0x50, required by TPS25750)

- External header for EEPROM programming

## 4.4 Anti-back-powering rules

- I2C_AON pull-ups to 3V3_AON only

- I2C_SYS_SW pull-ups to 3V3_SYS_SW only

- No device should be powered parasitically via SDA/SCL/GPIO.

- Isolation at domain intersections: 33–100Ω series resistors and/or I2C bus switch

- List all AON↔SYS nets and confirm the absence of crossed pull-ups

## 4.5 I2C Address Verification — Anti-collision

Summary of all I2C addresses used, by bus. No address should appear twice on the same bus.

| Bus | Component | Address | Fixed? | Conflict |
| --- | --- | --- | --- | --- |
| I2C_AON | BQ25792 | 0x6B | Yes (TI) | ✅ None |
| I2C_AON | MAX17049 | 0x36 | Yes (Maxim) | ✅ None |
| I2C_AON | LIS2DW12 | 0x19 | SA0=VDD | ✅ None |
| I2C_SYS_SW | MCP23017 | 0x27 | A0-A2=VDD | ✅ Outside the TPS range (0x20–0x23) |
| I2C_SYS_SW | IIS2ICLX | 0x6A | SDO=GND | ✅ None |
| I2C_SYS_SW | TPS25750 slave | 0x20–0x23 | ADCIN cfg | ✅ MCP at 0x27, no conflict |
| I2C_TPS | CAT24C256 | 0x50 | A0-A2=GND | ✅ Single (point-to-point) |

Result :By setting the MCP23017 to 0x27, all addresses on I2C_SYS_SW are guaranteed to be unique without depending on the ADCIN configuration of the TPS25750. TODO #8 (TPS25750 address verification) is closed.

Cross-bus note:IIS2ICLX (0x6A, I2C_SYS_SW) and BQ25792 (0x6B, I2C_AON) are on separate buses → no electrical conflict. However, do not change SDO/SA0 from IIS2ICLX to VDD (this would result in 0x6B, the same address as BQ25792 → confusion during debugging).

# 5. Sensors

## 5.1 LIS2DW12 — Low power wake

- Power supply: 3V3_AON

- Address: 0x19 (SA0 = VDD). Do not use 0x18 (SA0=GND) to avoid any ambiguity.

- INT1 → ESP32 RTC/LP GPIO (3V3_AON pull-up)

- Bus: I2C_AON

- Firmware: wake-only profile (low ODR / threshold interrupt)

## 5.2 IIS2ICLX — precision inclinometer

- Power supply: 3V3_SYS_SW (OFF in deep sleep)

- Address: 0x6A (SDO/SA0 pin 5 → GND). Do not set SA0=VDD (this would result in 0x6B = same address as BQ25792 on the other bus).

- Bus: I2C_SYS_SW

- Connect SB11/SB12

## 5.3 MAX17049 — fuel gauge 2S

Replacement:MAX17048 (1S) → MAX17049 (2S compatible).

Powered continuously by 3V3_AON, without gating MOSFET (continuous SoC monitoring).

# 6. Peripherals and connectors

## 6.1 RAK3172 — LoRaWAN

### 6.1.1 UART mapping

Use UART2 for host link:

| RAK Pin | Function | Connection | Net |
| --- | --- | --- | --- |
| Pin 1 (PA3) | UART2_RX | ← ESP32 UART_TX | RAK_RX |
| Pin 2 (PA2) | UART2_TX | → ESP32 UART_RX | RACK_TX |
| Pin 4 (PB6) | UART1_TX | NC (test pads) | — |
| Pin 5 (PB7) | UART1_RX | NC (test pads) | — |

### 6.1.2 LoRaWAN Strategy: Class A + periodic polling

Decision validated:The RAK3172 operates in LoRaWAN Class A. The ESP32 wakes up periodically via the RTC timer, activates the RAK, performs an uplink, listens to the RX1/RX2 windows to retrieve the pending downlinks, then everyone goes back to sleep.

| Setting | Value |
| --- | --- |
| LoRaWAN Class | Class A (RX only after TX) |
| Polling interval | Configurable firmware (default: 15 min, adjustable 1–60 min) |
| Cloud command latency | 0 to X min (= polling interval). The server buffers the downlinks. |
| RAK sleep current | ~1.5µA (STM32WL in stop2). AON budget compatible < 50µA. |
| RAK TX/RX current | ~30mA TX, ~6mA RX, for a few seconds per cycle. |

Justification:Class C (continuous RX ~6mA) would exceed the battery budget by x100. Class A polling preserves autonomy while allowing the reception of commands (door release, configuration, geolocation) with an acceptable delay.

### 6.1.3 RAK3172 power supply

Domain:3V3_AON (always powered). The RAK3172 must remain powered on at all times to maintain its LoRaWAN context (session keys, frame counters, ADR parameters).

- In ESP deep sleep mode: the RAK is in internal stop2 mode (~1.5µA). Its RFP (or custom AT) firmware manages sleep autonomously.

- Upon ESP wake-up: the ESP wakes up the RAK via UART (AT command or toggle of a wake pin). The RAK resumes operation within a few milliseconds.

- No rail interruption: if the RAK's power is cut, it loses its LoRaWAN session → a join is required every time it wakes up (~5–10s latency + consumption of a join request/accept). This should be avoided.

### 6.1.4 Pin IRQ (RAK_IRQ_N) — use in Class A

In Class A polling mode:The RAK_IRQ_N in the wired-OR IO7 is not the primary wake mechanism (that's the RTC timer). However, the IRQ line remains wired for:

- Future upgrade to Class C (if sufficient external/solar power supply)

- Reception of urgent messages outside the polling cycle (if the RAK receives a downlink while the ESP is asleep AND the RAK is in RX mode — marginal case in Class A)

- Diagnosis: Confirm that the RAK has received pending data.

RAK pin used for IRQ:To be defined (PA8, PA11, or other free GPIO pin on the RAK3172 → open-drain or push-pull output configured by RAK firmware). Connected to the wired-OR IO7 via a 100–330Ω series resistor. If the RAK does not support native hardware IRQs, an NPN/NMOS transistor can be used to convert a UART or GPIO signal to open-drain.

### 6.1.5 Scenario: receiving a LoRa command (e.g., open the latch)

Initial state:ESP in deep sleep, RAK in stop2 on 3V3_AON. The LoRa server has queued a downlink "open gate".

| Stage | Time | Action |
| --- | --- | --- |
| 1 | t=0 (RTC timer) | The RTC timer wakes the ESP32 from deep sleep (internal wake source, independent of the wired-OR IO7). |
| 2 | t ≈ boot | The ESP firmware wakes up the RAK3172 via UART (IO5/IO6). The RAK exits stop2 in a few milliseconds, with the LoRaWAN session intact. |
| 3 | t ≈ boot+ | The ESP commands the RAK to perform a Class A uplink (e.g., heartbeat, position, battery SoC). |
| 4 | t + ~1s (RX1) | The RAK opens the RX1 window. The server delivers the buffered "open gate" downlink. |
| 5 | t + ~2s | The RAK transmits the downlink to the ESP via UART. The firmware parses the command. |
| 6 | t + ~2.5s | The ESP activates 5V_EN (IO0 HIGH), waits for the +5V and 3V3_SYS_SW to stabilize, then writes MCP23017 GPB0 = HIGH via I2C_SYS_SW. The MOSFET switches → gate open. |
| 7 | t + ~5s | Firmware timeout: MCP23017 GPB0 = LOW. Door latch closed. The ESP cuts 5V_EN, puts the RAK back into sleep, and returns to deep sleep. |

Total latency:Between 0 and [polling interval] for the server to deliver the downlink (the command waits in a queue). Then ~2–3 seconds of hardware execution. With a 15-minute polling interval: worst case 15 minutes + 3 seconds.

If latency is unacceptable:The firmware can temporarily switch to Class C when the USB charger is plugged in (detected by the BQ25792 via I2C_AON). The RAK_IRQ_N line in the wired-OR then becomes fully relevant: downlink received instantly → IRQ → wake ESP → execution. The 3V3_AON current draw drops to ~8mA but is compensated by the USB power supply.

## 6.2 A7670E — GSM Modem

- Dedicated +3.8V rail (TPS63070 from +5V) — true power-off in deep sleep

- Control: GSM_PWRKEY_CTRL or toggle U20

- GNSS: 3V8 rail separated via U20, from +5V

- USIM1_DET / USIM2_DET: do not leave floating

## 6.3 TPS25750 — USB-PD

- I2C master (I2C_TPS): pins 16/17 → CAT24C256

- I2C slave (I2C_SYS_SW): pins 8/9 → controlled by ESP32

- BQ25792 CE#: 100kΩ pull-down (load ON by default). CP2102 CHREN connection removed (see §3.1).

## 6.4 Mechanical Modifications

| # | Element | Modification |
| --- | --- | --- |
| has | Slot GNSS F9P (H3) | Move or reverse the orientation of the u-blox slot |
| b | Connector J_MCP_GPB | Add B3B-XH-A for lines GPB1–GPB7 of MCP23017 (GPB0 reserved STRIKE_CTRL) |
| c | Battery connector P1 | Change to XH type connector |
| d | USB-C charge-only | Add dedicated XH connector for charging (+/- only) |
| e | RESET button (C19) | Change to B2B-XH-A(LF) |

## 6.5 Servo motor

Connector:3-pin header (JST-XH 2.54 type or equivalent)

Pinout:+5V / GND / PWM

| Setting | Specification |
| --- | --- |
| PWM control | Directly via the ESP32-C6 (not via shift register) |
| GPIO | IO1 = SERVO_PWM (V3 GPIO config) |
| Power supply | The +5V from the servo connector comes from the switched +5V rail → servo is only available when 5V_EN is ON |
| Domain | +5V (OFF in deep sleep) |

## 6.6 Solenoid electric strike (master key output)

Type of strike plate:DC 5V / 2A electromagnetic solenoid (fail-secure: locked when powered off, unlocked by pulse).

Field cabling:2 pairs of wires → 2 JST-SM connectors on the PCB.

| PCB Connector | Kind | Wires | Role |
| --- | --- | --- | --- |
| J_STRIKE_PWR | JST-SM male (2 pins) | Red + Black | Internal +5V output (TPS54531 rail switched) + GND. No external input. |
| J_STRIKE_SOL | JST-SM female (2-pin) | Blue + White | Output to solenoid gate (switched by MOSFET, 2A max). |

Control:MCP23017 GPB0 (I2C_SYS_SW bus) → N-channel MOSFET gate. The MCP provides the logic signal (~µA), the MOSFET switches the 2A power. The blue/white wires do NOT pass through the MCP.

Internal PCB wiring diagram:

- Red (+5V) → direct trace → Blue (solenoid +). Non-switched power path.

- White (solenoid return) → N-channel MOSFET drain

- MOSFET source → Black (GND power supply for door strike)

- Gate MOSFET ← MCP23017 GPB0 via R_gate 100Ω–1kΩ + pull-down 10kΩ–100kΩ (OFF by default)

- Flyback diode between Blue and White (cathode on the Blue side / V+, anode on the White side / drain)

| Setting | Specification |
| --- | --- |
| Control signal | MCP23017 GPB0 (address 0x27, GPIOB register bit 0) |
| MOSFET | N-ch logic-level, SOT-23 (e.g. AO3400A: Vgs(th) ≤1.4V, Id=5.7A @ Vgs=4.5V, Vds=30V). Sufficient 3.3V drive. |
| Solenoid current | 2A @ 5V internal (10W peak). Pulse mode only. 12V/24V door strikes NOT supported. |
| Maximum pulse duration | ≤ 5 seconds (beyond that: risk of overheating / coil destruction) |
| Flyback diode | SS34 (Schottky 3A/40V, SMA/DO-214AC). Comfortable margin for 2A pulsed current. SS14 (1A) should be avoided. |
| Default state | OFF (pull-down gate). Gate locked at boot / deep sleep / reset / MCP not powered. |
| Not at JST-SM | 2.50 mm (JST-SM standard) |

Integrated power stage on PCB (mandatory) The J_STRIKE_SOL output is NOT a direct GPIO output. It is switched by an N-channel logic-level MOSFET in low-side configuration, driven by MCP23017 GPB0 (STRIKE_CTRL). A flyback diode SS34 is placed as close as possible to the solenoid output (inductive clamp). A gate resistor R_gate (100Ω–1kΩ) in series and a pull-down R_pull-down (10kΩ–100kΩ) guarantee an OFF state by default (boot, reset, deep sleep, MCP unpowered). No relay required: the PCB includes a MOSFET power stage sized for the solenoid current; the MCP output never carries solenoid current.

TPS54531 constraint:The +5V rail must be able to supply a 2A peak current for the door strike in addition to other loads. Verify that the total +5V current budget remains within the limits of the TPS54531 (5A max continuous).

Firmware constraint:The MCP driver must implement a hardware or software timeout (watchdog) to cut off GPB0 after 3–5 seconds. Never leave the door strike continuously powered.

⚠️ Constraints / not supported:

• Output validated for 5V DC solenoid strike in pulsed mode (≤ 5 s). Max target current: 2A pulse (verify +5V budget TPS54531).

• 12V / 24V solenoids are NOT supported in this version. MOSFET Vds (AO3400A = 30V), flyback diode (SS34 = 40V), and connectors would need to be resized if a high-voltage strike is required.

• J_STRIKE_PWR is powered by the internal switched +5V rail (TPS54531). This is NOT an input for an external power supply.

# 7. Acceptance Criteria

Rules to check during the schematic/PCB V3 review:

| # | Ruler | Description |
| --- | --- | --- |
| 7.1 | Deep sleep by default | Only 3V3_AON ON. All other rails physically OFF. |
| 7.2 | Explicit domains | 3V3_AON = wake + supervision. 3V3_SYS_SW = run + peripheral. GSM = dedicated. |
| 7.3 | Single controller | ESP32 = sole controller. No peripheral device is self-powered. |
| 7.4 | Determinism | USB hot-plug, battery insertion → no undefined state or reset. |
| 7.5 | Wake sources | All on GPIO deep-sleep capable, polarity and pulls defined. |
| 7.6 | Anti-back-powering | Pull-ups on their respective domain. Insulation at crossings. |
| 7.7 | GSM robustness | Rail OFF in sleep. Bursts do not disturb logic rails. |

Confirm compliance with 7.1–7.7 and report any deviations.

# 8. ESP32-C6 GPIO Assignment

Final GPIO listing for V3. Only IO0–IO7 (LP_GPIO) can wake the ESP32 from deep sleep.

⚠️ Strapping pins ESP32-C6:The following pins are sampled at reset for boot configuration: GPIO4 (MTMS/JTAG), GPIO5 (MTDI/JTAG), GPIO8, GPIO9, GPIO15 (JTAG signal source). Ref. ESP32-C6 Datasheet §3.3 “Strapping Pins”.

| GPIO | Strapping | Our V3 usage | Analysis |
| --- | --- | --- | --- |
| IO4 | MTMS (JTAG TMS) | LIS2_INT_WAKE | ✅ OK: Low internal pull-up. LIS2DW12 INT1 is open-drain active low with serial R → does not affect boot. JTAG not used. |
| IO5 | MTDI (JTAG TDI) | RAK_UART_TX | ✅ OK: Floating by default. The RAK3172 RX is high impedance → no power at boot. Check that the UART line is not pulling low on reset. |
| IO8 | Boot mode | Avoided | ✅ Not used. |
| IO9 | Boot mode (pull-up) | Avoided | ✅ Not used. |
| IO15 | JTAG src select | I2C_SYS_SW_SDA | ✅ OK: I2C pull-up (3V3_SYS_SW) = HIGH at boot. GPIO15 HIGH = default JTAG source (no impact). If 3V3_SYS_SW is off at boot, the pin floats → OK as well (default value accepted). |

Reset with SYS_SW ON:If the ESP32 resets while 3V3_SYS_SW is active, the I2C pull-up forces IO15 to HIGH during the jumper connection. GPIO15 HIGH = "JTAG signal source: default". This behavior is confirmed to be acceptable (ESP32-C6 Datasheet §3.3). No corrective action is necessary, but documenting it prevents false alarms during review.

Recommended layout — strapping protection:IO4 (LIS2_INT) and IO5 (RAK_UART_TX): 1k–10kΩ series resistance is OK (slow signals, no impact). IO15 (I2C_SYS_SW_SDA): 22–100Ω series resistance MAXIMUM. A higher value degrades I2C edges at 400kHz (awful rise time with 10kΩ series + pull-up). General rule: I2C SDA/SCL = 22–100Ω series max; IRQ/GPIO = 1k–10kΩ OK.

## 8.1 AON Domain (always powered)

| GPIO | Signal | Function | LP_GPIO |
| --- | --- | --- | --- |
| IO10 | I2C_AON_SCL | I²C AON (BQ25792, MAX17049, LIS2DW12) | No |
| IO11 | I2C_AON_SDA | I²C AON | No |
| IO4 | LIS2_INT_WAKE | Vibration wake (LIS2DW12 INT1) | Yes (LP4) |
| IO7 | WAKE_OR_N | Wired-OR active low: VBUS_DET + BTN + RAK_IRQ (see §8.6) | Yes (LP7) |
| IO5 | RAK_UART_TX | ESP → RAK3172 (UART2_RX) | Yes (LP5) |
| IO6 | RAK_UART_RX | RAK3172 → ESP (UART2_TX) | Yes (LP6) |

Total AON:7 GPIOs. Wake LP_GPIO: IO4 (vibration), IO7 (wired-OR 3 sources) + internal RTC timer.

## 8.2 Domain SYS / Runtime

### I2C_SYS_SW (IO3 / IO15)

Multi-drop bus for runtime devices. IO3 freed up by removing BQ_INT (replaced by VBUS_DET in the wired-OR). IO8/IO9 bypassed.

| GPIO | Signal | Function |
| --- | --- | --- |
| IO3 | I2C_SYS_SW_SCL | I²C SYS (MCP23017, IIS2ICLX, TPS25750 slave) |
| IO15 | I2C_SYS_SW_SDA | I²C SYS (pull-ups on 3V3_SYS_SW) |

### microSD (dedicated SPI)

| GPIO | Signal | Function |
| --- | --- | --- |
| IO20 | SD_SCK | SPI clock |
| IO18 | SD_MOSI | SPI data out |
| IO21 | SD_MISO | SPI data in |
| IO19 | SD_CS | SPI chip select |

### GNSS (dedicated UART)

| GPIO | Signal | Function |
| --- | --- | --- |
| IO12 | GNSS_RX | ESP ← GNSS |
| IO13 | GNSS_TX | ESP → GNSS |

### GSM (dedicated UART)

| GPIO | Signal | Function |
| --- | --- | --- |
| IO22 | GSM_RX | ESP ← A7670E |
| IO23 | GSM_TX | ESP → A7670E |

### Debug UART (preserved on ESP)

| GPIO | Signal | Function |
| --- | --- | --- |
| IO24 | PROG_RX | Debug UART RX (CP2102) |
| IO25 | PROG_TX | Debug UART TX (CP2102) |

## 8.3 V3 Controls (power + actuators)

| GPIO | Signal | Function | LP_GPIO |
| --- | --- | --- | --- |
| IO0 | 5V_EN | Active buck +5V (TPS54531), hardware pull-down | Yes (LP0) |
| IO1 | SERVO_PWM | Servo PWM signal | Yes (LP1) |
| IO2 | SPARE | Libre (formerly STRIKE_CTRL, relocated to MCP23017 GPB0) | Yes (LP2) |

## 8.4 MCP23017 — IO Expander (SYS_SW)

The MCP23017 offloads all slow and non-critical commands from the ESP32 at boot time. It is powered by 3V3_SYS_SW, on the I2C_SYS_SW bus (IO3/IO15), address 0x27 (A0=A1=A2=VDD). RESET pull-up to 3V3_SYS_SW.

### Recommended outings (GPA)

| MCP Pin | Signal | Function |
| --- | --- | --- |
| GPA0 | GSM_EN | Enable A7670E power |
| GPA1 | GNSS_EN | Enable GNSS power |
| GPA2 | SD_EN | microSD power gate (if applicable) |
| GPA3 | GSM_PWRKEY | A7670E power-on pulse |
| GPA4 | GSM_RESET | A7670E reset |
| GPA5 | GNSS_RESET | GNSS reset |
| GPA6 | LED_STATUS | system status LED |
| GPA7 | SPARE_MCP_A | Reserve |

### Reserve and Actuators (GPB)

| MCP Pin | Signal | Function |
| --- | --- | --- |
| GPB0 | STRIKE_CTRL | MOSFET gate control for solenoid strike (5V/2A pulsed). See §6.6. |
| GPB1–GPB7 | SPARE_MCP_B[1..7] | 7 lines available for future signals |

Boot sequence:ESP32 turns on SYS_SW / +5V → MCP23017 starts → ESP configures MCP outputs via I2C_SYS_SW → MCP manages the "small switches".

MCP Interrupt (optional):Connect INTA/INTB to an ESP pin if you want to report input changes without polling. Not required at the moment (no obvious pin without falling back to IO8/IO9).

## 8.5 Summary

| Criteria | Value |
| --- | --- |
| ESP32 GPIO used | 20 (IO0–IO1, IO3–IO7, IO10–IO13, IO15, IO18–IO25). IO2 = spare (LP2). |
| ESP32 free GPIOs | IO2 (LP2), IO8/IO9 (strapping), IO14, IO16, IO17 (if routed) |
| GPIO MCP used | GPA0–GPA7 (7 + 1 spare) + GPB0 (STRIKE_CTRL) |
| MCP free GPIOs | GPB1–GPB7 (7 available) |
| ESP shared buses | None (except wired-OR IO7 = 3 sources) |
| Multiplexing | None |
| Wake pins (LP_GPIO) | IO4 (LIS2_INT), IO7 (VBUS_DET + BTN + RAK_IRQ) |

ESP32 spares note:IO14, IO16, and IO17 are free on the chip but may not be routed to the WROOM module. Check the pinout of the module being used before counting them as guaranteed spares.

## 8.6 Wired-OR IO7 detail: VBUS_DET + BTN_AON + RAK_IRQ

Principle:IO7 (LP_GPIO7) is shared in a wired active-OR low configuration between three sources. A single pull-up resistor connects to 3V3_AON (~10kΩ). Any of these (USB, button, LoRa) can wake the ESP with a stable level.

| Source | Output type | R series | Active |
| --- | --- | --- | --- |
| VBUS_DET_N | Open-drain comparator (~10nA) | 100–330Ω | LOW |
| BTN_N | NO contact to GND | 100–330Ω | LOW |
| RAK_IRQ_N | Open-drain (or via NPN/NMOS) | 100–330Ω | LOW |

At rest: IO7 = HIGH (pull-up 3V3_AON). Wake: IO7 → LOW as soon as a source pulls the line to GND.

Identification after wake-up:The firmware reads the BQ25792 register (via I2C_AON) to detect VBUS, queries the RAK3172 (via UART) for a LoRa event, and performs a button debounce. If neither BQ nor RAK has an event, the button is debounced.

# 9. Open items

| # | Item | Required action |
| --- | --- | --- |
| 1 | Bead L11 (formerly L4) | Confirm 3A=Irms continuous, 6A=Isat. Provide P/N + datasheet. |
| 2 | Nets cross AON↔SYS | List all nets, confirm pulls, add series resistors. |
| 3 | Spares IO14/IO16/IO17 | Check the pinout of the ESP32-C6 module being used: are these GPIOs routed? |
| 4 | GNSS Test A7670E | Confirm GNSS + GSM functional. |
| 5 | Supply A7670 direct SYS? | Decide: buck SYS→3.8V or keep +5V→3.8V. |
| 7 | RAK3172 IRQ Pin | Determine which GPIO RAK (PA8, PA11?) generates RAK_IRQ_N. Check if native open-drain or NPN is required. |
| 8 | I2C Address TPS25750 | ✅ Resolved: MCP23017 moved to 0x27 (A0-A2=VDD), outside the range of TPS25750 (0x20–0x23). No more possible conflicts. |
| 6 | CP2102 domain | ✅ Powered from VBUS (VREGIN). Independent of 3V3_SYS_SW for dead ESP flash. Validated. |

## Validated decisions (previous open points)

| Item | Decision |
| --- | --- |
| Wake USB | ✅ Discrete VBUS_DET in wired-OR IO7. BQ_INT removed (IO3 freed for I2C_SYS_SW). |
| I2C_SYS_SW pins | ✅ IO3 (SCL) + IO15 (SDA). Avoids IO8/IO9 (strapping). |
| BQ25792 I2C bus | ✅ BQ25792 on I2C_AON (IO10/IO11). Accessible from wake, before SYS_SW. |
| BTN_AON + RAK_IRQ | ✅ Wired-OR 3 sources on IO7 (VBUS_DET + BTN + RAK_IRQ), active low. |
| MCP23017 | ✅ On I2C_SYS_SW (0x27, A0-A2=VDD). Out of TPS25750 range (0x20–0x23) → guaranteed anti-collision. |
| Debug UART | ✅ Stored on ESP (IO24/IO25). Not relocated to MCP. |
| STRIKE_CTRL | ✅ Offloaded to MCP23017 GPB0 (MOSFET gate). IO2 ESP released as spare (LP2). |
| CP2102 domain | ✅ Powered from VBUS (VREGIN pin 7). Flashing possible with dead/virgin ESP. |
| microSD cutout | ✅ Load switch high-side (VDD_SD). No low-side MOSFET (anti back-powering). |
| LoRa Strategy | ✅ Class A + periodic polling (RTC timer, 15 min default). RAK in stop2 on 3V3_AON. Class C possible when USB connected. |